# 0195p DTAC tactile indicators and stair edgings

Branded worksection

This branded worksection Template has been developed by NATSPEC in conjunction with DTAC Pty Ltd (the Product Partner) and may be used whilst the Product Partner is licensed to distribute it. The copyright remains with NATSPEC. As with all NATSPEC worksections, it is the responsibility of the user to make sure it is completed appropriately for the project. The user should also review its applicability for local conditions and regulations. Check www.natspec.com.au for the latest updated version.

Worksection abstract

This branded worksection Template is applicable to the supply and installation of DTAC tactile indicators, edge protectors, and edging for stair nosings.

How to use this worksection

Customise this worksection Template for each project. See A guide to NATSPEC worksections (www.natspec.com.au) for information on Template structure, word styles and completing a worksection.

Related material located elsewhere in NATSPEC

If a listed worksection is not part of your subscription package and you wish to purchase it, contact NATSPEC.

Related material may be found in other worksections, including:

0315 Concrete finishes for concrete stairs and surface finishes.

0541 Access floors.

0551 Joinery for timber stairs.

0552 Metalwork - fabricated for steel stairs.

0613 Terrazzo in situ for surface finish.

0631 Ceramic tiling for tiled floors and stairs.

0632 Stone and terrazzo tiling for tiled floors and stairs.

0651 Resilient finishes for stair and floor finish.

0652 Carpets for stair and floor finish.

0654 Multilayered board flooring for stair and floor finish.

0655 Timber flooring for stair and floor finish.

0657 Resin based seamless flooring.

Documenting this and related work

You may document this and related work as follows:

Locate tactile indicators, edge protectors and stair edgings on drawings.

If using this worksection, remove tactile indicators from the other schedules as appropriate.

The Normal style text of this worksection may refer to items as being documented elsewhere in the contract documentation. Make sure they are documented.

Search acumen.architecture.com.au, the Australian Institute of Architects' practice advisory subscription service, for notes on the following:

Guarantees and warranties.

Slip resistance compliance and testing.

Slip resistance design considerations.

Specifying ESD

DTAC manufacture a tactile indicator featuring a cupped underside, using less stainless steel, making it more sustainable.

Refer to NATSPEC TECHreport TR 01 on specifying ESD.

## GENERAL

DTAC is an Australian company with over 15 years' experience in design and manufacturing excellence, all backed by industry leading support. DTAC comprises of a specialist team of professionals that prides itself on offering aesthetically pleasing, BCA compliant, architectural tactile ground surface indicators, stair nosing and edging, urban landscape edge protection products and more. DTAC's unequalled attention to detail enables architects, designers and builders to make the right choice for aesthetic and functional conformance in every project.

### RESPONSIBILITIES

#### General

Requirement: Provide DTAC tactile indicators, stair nosing and edging, threshold transition strips, urban edge protectors and handrail tactile indicators, as documented.

Documented is defined in 0171 General requirements as meaning contained in the contract documents.

### COMPANY CONTACTS

#### DTAC technical contacts

Website: www.dtac.com.au/contact

### CROSS REFERENCES

#### General

Requirement: Conform to the following:

0171 General requirements.

0171 General requirements contains umbrella requirements for all building and services worksections.

List the worksections cross referenced by this worksection. 0171 General requirements references the 018 Common requirements subgroup of worksections. It is not necessary to repeat them here. However, you may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

NATSPEC uses generic worksection titles, whether or not there are branded equivalents. If you use a branded worksection, change the cross reference here.

### STANDARDS

#### General

Tactile indicators: To AS/NZS 1428.4.1 (2009).

The NCC cites AS 1428.4 (1992) and AS/NZS 1428.4.1 (2009). The current edition is AS/NZS 1428.4.1 (2009).

Stair edging: To AS 1428.1 (2021).

The NCC cites AS 1428.1 (2001) and AS 1428.1 (2021). The current edition is AS 1428.1 (2021).

### MANUFACTURER'S DOCUMENTS

#### Technical manuals

DTAC Tactile ground surface indicators (TGSI):

Warning tactiles: dtac.com.au/product‑category/warning‑tactiles

Directional tactiles: dtac.com.au/product‑category/directional‑tactiles

Integrated tactiles: dtac.com.au/product‑category/integrated‑tactiles

Carpet tactile systems: dtac.com.au/product‑category/carpet‑tactile‑systems

DTAC PEMKO® Stair treads nosing and edging, threshold transitions and edging strips:

Stair nosing and edging: dtac.com.au/product‑category/stair‑nosing‑&edging

Modular stair edging: dtac.com.au/product‑category/modular‑stair‑edging

Thresholds and transition strips: dtac.com.au/product‑category/thresholds‑&transition‑strips

DTAC Urban edge protectors: dtac.com.au/product‑category/urban‑edge‑protectors

DTAC Handrail tactile indicators (HRTIs): dtac.com.au/product‑category/handrail‑tactile‑indicators

### SUBMISSIONS

#### Products and materials

Type tests: Submit test results to PRODUCTS, GENERAL, Tests for the following:

Slip resistance of tactile indicators and edgings.

The DTAC range of tactile indicators, stair nosing and edgings are tested to a range of slip resistance tests. According to the Wet Pendulum Slip rating AS 4586 (2013), Appendix A completed in a NATA accredited laboratory, all DTAC warning TGSI products meet a P5 slip resistance classification. All DTAC directional TGSI, stair nosing and edging products meet a P4 to P5 slip resistance classification.

Luminance reflectance of tactile indicators and edgings.

Type tests are carried out off site. However, submission of evidence of a successful type test may be called up here for requirements specified in PRODUCTS.

Prototypes

Requirement: Submit prototypes to EXECUTION, GENERAL, Prototypes.

Include this Optional style subclause by changing to Normal style text if the Optional EXECUTION, GENERAL, Prototypes subclause is included.

#### Tests

Detail the tests required in EXECUTION and list the submissions required here.

Completion tests: Submit results of the following:

Slip resistance.

Luminance contrast.

Site test submissions correspond to the testing requirements in EXECUTION, TESTING, Completion tests. If testing requirements are deleted from EXECUTION, remove submission requirements above.

#### Warranties

Requirement: Submit warranties to COMPLETION, Warranties.

### INSPECTION

#### Notice

Inspection: Give notice so that inspection may be made of the completed substrate ready for tactile indicators, and nosing and edging installation.

A site inspection can be conducted before installation.

Edit to suit the project, adding critical stage inspections required.

Hold points, if required, should be inserted here.

## PRODUCTS

### GENERAL

#### Product substitution

Other products: Conform to SUBSTITUTIONS in 0171 General requirements.

SUBSTITUTIONS in 0171 General requirements sets out the submissions required if the contractor proposes alternative products. Refer also to NATSPEC TECHnote GEN 006 for more information on proprietary specification.

Avoid the use of phrases such as ‘or equivalent' for substitutions of proprietary products, as products that look or claim to be equivalent may not have undergone the same testing and certification processes as the specified products, and therefore may not be able to fulfill the same performance requirements.

DTAC products have been tested by NATA Accredited Testing Laboratories to Australian and New Zealand, UL, ANSI, BHMA, European, British and ISO standards. Accredited Testing Laboratories include International Door & Window Laboratories (IDWL), CSIRO, BRANZ, Jensen Hughes (formerly Warringtonfire) and UL.

DTAC choose to issue reports from International Door & Window Laboratories (IDWL), a NATA Accredited Testing Laboratory, as evidence of suitability for use to NCC (2025) A5G1.

#### Tests

0171 General requirements defines different tests in INTERPRETATION, Definitions.

Slip resistance of tactile indicators and edgings: To AS 4586 (2013).

The DTAC range of tactile indicators and edging are tested to a range of slip resistance tests including AS 4586 (2013) Appendices A, C & D achieving a P4 to P5 rating.

Refer to the DTAC product Technical Data sheets for product specific test results.

Luminance reflectance of tactile indicators and edgings: To AS/NZS 1428.4.1 (2009) Appendix E and AS 1428.1 (2021) Appendix B.

The NCC cites AS 1428.4 (1992) and AS/NZS 1428.4.1 (2009). The current edition is AS/NZS 1428.4.1 (2009).

The NCC cites AS 1428.1 (2001) and AS 1428.1 (2021). The current edition is AS 1428.1 (2021).

Product luminance test results are included on the DTAC technical data sheets, which are presented by product type.

### DTAC TACTILE INDICATORS, STAIR NOSING AND EDGING, THRESHOLD AND TRANSITION STRIPS, URBAN EDGE PROTECTORS AND HANDRAIL TACTILE INDICATORS

DTAC can provide technical advice so that the appropriate tactile and edging products are specified.

#### Warning Tactile products

DTAC 'Warning Tactile' products are identifiable by the unique registered ‘double lip' design, providing high slip resistance performance.

Classic: Tactile indicators with a concentric circle design machined or injection moulded on the horizontal face and a smooth outer edge.

Available in Type 316 stainless steel (clear or black PVD coated) along with black silicon carbide top (insert), solid brass, or in black, yellow, light grey or white UV‑stabilised thermoplastic urethane (TPU). Also available in Type 316 stainless steel outer edge with black, yellow or white UV‑stabilised thermoplastic urethane (TPU) top or black silicon carbide top. NOTE: Type 316 stainless steel and solid brass products are also available in ‘flat‑back' version.

Terraced: Tactile indicators with a concentric circle design machined or injection moulded on the horizontal face and a terraced slip‑resistant outer edge.

Available in Type 316 stainless steel (clear), solid brass, or Type 316 stainless steel outer edge with a black UV‑stabilised thermoplastic urethane (TPU) top. NOTE: Type 316 stainless steel and solid brass products are also available in ‘spiked or flat' versions.

Ecotac® Classic: Tactile indicators with a cupped underside, a concentric circle design machined on the horizontal face and a smooth outer edge.

Ecotac® Terraced: Tactile indicators with a cupped underside, a concentric circle design machined on the horizontal face and a terraced slip‑resistant outer edge.

Available in Type 316 stainless steel (clear or black PVD coated) and solid brass.

#### Directional Tactile products

Cross‑Hatch 'Directional' grooved tactile: Tactile indicator with a grooved design and alternate slots machined on the horizontal face and a smooth outer edge.

Available in Type 316 stainless steel, integrated on a Type 316 stainless steel base.

Classic ‘Directional' with carborundum tactile: Tactile indicator with silicon carbide inserted and a smooth outer edge.

Available in Type 316 stainless steel (clear or black PVD coated) and also with a black silicon carbide top.

Classic: UV‑stabilised thermoplastic urethane (TPU) injection moulded tactile with an undulating groove design on the horizontal face and a smooth outer edge.

Available in black, yellow, light grey or white. Custom colours are also available, subject to minimum quantities.

#### Integrated Tactile products

Classic ‘Warning' tactile: An integrated Type 316 stainless steel plate with Classic warning tactile indicators on the horizontal face.

Available in Type 316 stainless steel integrated on a Type 316 stainless steel base (clear or black PVD coated). Variant sizes available.

Classic ‘Warning' Black Top tactile: An integrated Type 316 stainless steel plate with Classic warning black tactile indicators on the horizontal face.

Available in Type 316 stainless steel outer edge with a black silicon carbide top on a Type 316 stainless steel base. Variant sizes available.

Cross‑Hatch ‘Directional' grooved tactiles: An integrated Type 316 stainless steel plate with a grooved design and alternate slots machined on the horizontal face and a smooth outer edge.

Available in Type 316 stainless steel, integrated on a Type 316 stainless steel base.

Fibreglass Reinforced ‘Warning' tactiles: An integrated UV stabilised fibreglass/composite moulded tactile with Classic warning tactile indicators on the horizontal face.

Available in UV stabilised fibreglass/composite material in black, yellow or white. Also supplied with SS screws.

Fibreglass Reinforced ‘Directional' tactiles: An integrated UV stabilised fibreglass/composite moulded tactile with Directional tactile indicators on the horizontal face.

Available in UV stabilised fibreglass/composite material in black, yellow or white. Also supplied with SS screws.

Ultimat® Tactile: An integrated UV‑stabilised thermoplastic urethane (TPU) injection moulded tactile with Classic tactile indicators on the horizontal face.

Available in UV‑stabilised thermoplastic urethane (TPU) top/base in black, yellow, light grey or white. Custom colours are also available, subject to minimum quantities.

Ultimat Duo® Tactile: An integrated UV‑stabilised thermoplastic urethane (TPU) injection moulded tactile with contrasting coloured Classic tactile indicators on the horizontal face.

Available in UV‑stabilised thermoplastic urethane (TPU) top & nylon base in variant colours dependent on project requirements. Consult DTAC. Custom colours are also available, subject to minimum quantities.

#### Carpet Tactile systems

DTAC Carpet tactile systems comprise of variant screwable tactile indicators, which are affixed through the carpet onto steel carpet plates that are adhered on to the primary substrate.

Classic (Low/High Pile): Tactile indicators with a concentric circle design machined on the horizontal face, a smooth outer edge and a female thread stud to affix to the carpet plate.

Available in Type 316 stainless steel (clear) or solid brass. Variant sizes available to suit Low or High carpet pile.

Terraced (Low/High Pile): Tactile indicators with a concentric circle design machined on the horizontal face, a terraced slip‑resistant outer edge and a female thread stud to affix to the carpet plate.

Available in Type 316 stainless steel (clear). Variant sizes available to suit Low or High carpet pile.

Ecotac® Classic (High Pile): Tactile indicators with a cupped underside, a concentric circle design machined on the horizontal face, a smooth outer edge and an extended shaft. This product does not require steel carpet plates.

Available in Type 316 stainless steel (clear).

Steel Carpet Plates: Underflooring steel plate with threaded inserts to permit Carpet Tactiles to be affixed.

Available in Type 316 stainless steel (clear).

#### Stair Nosing and Edging products

More than 25 µm thick (black and clear anodise only), the DTAC hard anodising process produces a tough protective coating of aluminium oxide on the surface of the aluminium. This results in a uniform, hard wearing, scratch resistant surface ideal for commercial applications. Also included are a range of Anti‑slip tapes with a durable, chemical and oil resistant PVC tape with an 80 grit silicon carbide top surface and fibreglass edging with high slip resistance and durability.

PEMKO® ‘Modular ‑ Carpet' edging: Extruded aluminium sections with provision to insert any of the below edging into it (Corduroy, Pinstripe etc).

Anodised finishes available include natural and black. Sections also feature high strength double‑sided tape for application of PEMKO Corduroy, Pinstripe, Pleat and stitch edging. Supplied in lengths of 3.2 m.

PEMKO® ‘Modular ‑ Surface' edging: Extruded aluminium sections with provision to insert any of the below edging into it (Corduroy, Pinstripe etc).

Anodised finishes available include natural and black. Sections also feature high strength double‑sided tape for application of PEMKO Corduroy, Pinstripe, Pleat and Stitch edging. Supplied in lengths of 3.2 m.

PEMKO® Corduroy edging: Extruded aluminium sections with a continuous corduroy pattern.

Anodised finishes available include natural, black or gold. Sections available flat or angled. Supplied in lengths of 2.5 or 3.2 m.

PEMKO® Pinstripe edging: Extruded smooth, blunt edged aluminium sections with an intermittent high and low profile.

Anodised finishes available include natural and black. Sections available in flat or angled. Supplied in lengths of 2.5 or 3.2 m.

PEMKO® Pleat edging: Extruded smooth, aluminium sections with an intermittent continuous corduroy pattern.

Anodised finishes available include natural or black. Sections available in flat or angled. Supplied in lengths of 2.5 or 3.2 m.

PEMKO® Stitch edging: Extruded aluminium sections with a continuous stitch pattern.

Anodised finishes available include natural and black. Sections available in flat or angled. Supplied in lengths of 2.5 or 3.2 m.

PEMKO® Suede edging: Extruded anodised aluminium sections with a continuous band of silicon carbide.

Anodised finishes available in natural or black. Sections available in flat or angled. Supplied in lengths of 3.2 m.

PEMKO® Corduroy Carpet edging: Extruded anodised aluminium sections with a continuous corduroy pattern and inclusion of escutcheon plates.

Anodised finishes available in natural, black or gold. Sections available in flat or angled. Supplied in lengths of 2.5 or 3.2 m.

PEMKO® Urban edging: Continuous band of silicon carbide inserted into anodised aluminium extrusions.

Anodised finishes available in natural anodised with black, grey, yellow or white silicon carbide infill or black anodised with black or white silicon carbide infill. Sections available in flat, angled or for carpet. Suitable for all stable finished outdoor or indoor surfaces and substrates. Supplied in lengths of 2.5 or 3.2 m.

PEMKO® Rugged edging: Continuous band of fibreglass reinforced plastic extrusion.

Available in angled profiles, in black, yellow and white. Supplied in lengths of 3.2 m.

#### Thresholds and Transition Strips

Over 25 µm thick (black and clear anodise only), the DTAC hard anodising process produces a tough protective coating of aluminium oxide on the surface of the aluminium. This results in a uniform, hard wearing, scratch resistant surface ideal for commercial applications. These products are ideal in conjunction with door bottom seals. (Refer to Raven products – www.raven.com.au).

PEMKO® Threshold Plates: Extruded aluminium sections, many featuring a ribbed profile and integrated gasket sills.

Anodised finishes available in natural and several in black (identified by 'BK"). Some products include integrated PVC sill gaskets. Sections available in variant sizes 1.0, 2.0 and 3.0 mm.

#### Urban Edge Protectors

Round button: 35 mm diameter urban edge protector featuring round edge.

Available in Type 316 stainless steel or solid brass.

Bevelled button: 35 mm diameter urban edge protector featuring bevelled edge.

Available in Type 316 stainless steel or solid brass.

Thin bar: 5 mm thick x 15 mm wide x 110 mm long.

Available in Type 316 stainless steel only.

Thick bar: 10 mm thick x 15 mm wide x 110 mm long.

Available in Type 316 stainless steel only.

Bullnose: 10 mm thick x 15 mm wide x 110 mm long with 30 mm front return.

Available in Type 316 stainless steel only.

#### Handrail Tactile Indicators

Handrail button: Machined domed Type 316 stainless steel button.

Available in Type 316 stainless steel only.

## EXECUTION

### GENERAL

Prototypes

Requirement: Provide a prototype of the DTAC product installed in the finished substrate.

Location and extent:

Preferably show the location and extent on the drawings.

Incorporation: Subject to approval, incorporate the prototype in the completed works.

If a prototype is a project requirement, consider including this Optional style text by changing to Normal style text.

#### Substrate preparation and installation of DTAC products

Requirement: To DTAC's recommendations and fitting instructions.

Location: As documented.

'Classic and Terraced Warning' Tactile and Directional Indicators:

Stone, masonry, timber or vinyl substrates: Drill and pressure fit.

Vitrified porcelain, ceramics, glass or metal substrates:

Select Diamond core/pressure fit or Direct stick from the options in the DTAC technical data sheet.

Carpet or carpet tile substrates: Drill and screw.

Handrail Tactile Indicators: Drill and glue to substrate.

Ultimat® and Ultimat Duo® Tactile: Direct stick to substrate (excluding asphalt/bitumen) with DTAC tactile adhesive.

Urban Edge Protectors:

Stone, masonry, timber or vinyl substrates: Drill and glue.

Vitrified porcelain, ceramics, glass or metal substrates: Diamond core drill and glue.

#### Stair Nosing and Edging, Threshold and Transition products

PEMKO® Corduroy, Pinstripe, Pleat, Stitch, Suede, Urban and Rugged fibreglass threshold plates:

Stone, masonry, timber or vinyl substrates: Adhesive fix.

Vitrified porcelain, ceramics, glass or metal substrates: Adhesive fix.

Carpet or carpet tile substrates: Adhesive fix.

PEMKO® Modular Carpet and Surface Edging System:

Stone, masonry, timber or vinyl substrates: Adhesive fix / Screw Fix.

Vitrified porcelain, ceramics, glass or metal substrates: Adhesive fix / Screw Fix.

Carpet or carpet tile substrates: Adhesive fix / Screw Fix.

### TESTING

0171 General requirements defines different tests in INTERPRETATION, Definitions and calls for an inspection and testing plan in TESTING ‑ GENERALLY, Inspection and testing plan.

#### Completion tests

Slip resistance of completed installation: To AS 4663 (2013).

Delete if not required. See NATSPEC TECHnote DES 001. The wet‑barefoot inclining platform test and the oil‑wet inclining platform test cannot be performed in situ.

Luminance contrast testing of completed installation: Submit evidence of conformity to AS/NZS 1428.4.1 (2009) Appendix E and AS 1428.1 (2021) Appendix B.

The NCC cites AS 1428.4 (1992) and AS/NZS 1428.4.1 (2009). The current edition is AS/NZS 1428.4.1 (2009).

The NCC cites AS 1428.1 (2001) and AS 1428.1 (2021). The current edition is AS 1428.1 (2021).

DTAC can test substrates for luminance contrast to make sure the installed DTAC tactile indicators and edging conform to AS/NZS 1428.4.1 (2009) Appendix E for tactile indicators and AS 1428.1 (2021) Appendix B for edging. Delete if not required.

### COMPLETION

#### Warranties

Refer to 0171 General requirements for appropriate warranty type and the terms covered in the warranty.

Type:

Supplier's warranty: Warranty to cover manufacturing defects and defects with products and materials delivered to site (Supply Only).

Supplier's and Installer's interlocking warranty: Interlocking warranty to cover manufacturing defects and defects with products and materials delivered to site, including their application or installation (Supply & Install only).

Period: 2 years.

The DTAC range of tactile indicators and edging are guaranteed for 2 years against defects in materials and workmanship, provided the products are fitted and inspected to the manufacturer's specifications.

## SELECTIONS

Schedules are a tool to specify properties required for products or systems. If the principal permits documentation of the product or system by proprietary name, some of the properties may be unnecessary and can be deleted. Document the product or system's location or application here and/or on the drawings with a matching project code. Refer to NATSPEC TECHnote GEN 024 for guidance on using and editing schedules.

### DTAC PRODUCTS

#### Tactile indicator and edge protector schedule

|  | TI1 | TI2 | TI3 |
| --- | --- | --- | --- |
| Type |  |  |  |
| Product code |  |  |  |
| Slip‑resistance classification |  |  |  |
| Substrate |  |  |  |

The codes in the header row of the schedule designate each application or location of the item scheduled. Edit the codes to match those in other contract documents.

Type: Select from Warning tactile, Directional tactile, Integrated tactile or Carpet tactile.

Product code: Refer to the DTAC technical data sheets.

Nominate the required product code from the options below. For multiple selections, edit table to suit.

Warning Tactile products

Classic: Select from:

DT0101 Tactile Flat SS.

DT0103 Tactile Carpet SS 3 mm.

DT0106 Tactile Carpet SS 6 mm.

DT0123 Tactile PVD Black Flat SS.

DT0200 Tactile Spike Insert Blacktop SS.

DT0200W Tactile Spike Insert Whitetop SS.

DT0200Y Tactile Spike Insert Yellowtop SS.

DT0201 Tactile Flat Insert Blacktop SS.

DT0201W Tactile Flat Insert Whitetop SS.

DT0212 Tactile Classic Spike Carborundum Black SS.

DT0213 Tactile Classic Flat Carborundum Black SS.

DT0301 Tactile Flat Brass.

DT0400Y Tactile Spiked Colourmax Yellow.

DT0600B Tactile Spiked Urethane Black.

DT0600LG Tactile Spiked Urethane Light Grey.

DT0600W Tactile Spiked Urethane White.

DT0600Y Tactile Spiked Urethane Yellow.

Terrace: Select from:

DT0110 Tactile Spike Terraced SS.

DT0111 Tactile Flat Terraced SS.

DT0210 Tactile Spike Insert Terraced Blacktop SS.

DT0211 Tactile Flat Insert Terraced Blacktop SS.

DT0310 Tactile Spike Terraced Brass.

DT0311 Tactile Flat Terraced Brass.

Ecotac® Classic: Select from:

DT0120 Ecotac Tactile Spike SS.

DT0122 Ecotac PVD Black Tactile Spike SS.

DT0125 Ecotac Carpet 25 mm Tactile Spike SS.

DT0312 Ecotac Tactile Classic Spike Brass.

Ecotac® Terraced: Select from:

DT0130 Ecotac Tactile Spike Terraced SS.

Directional Tactile products

Cross‑Hatch 'Directional' grooved tactile: Select from:

DIR0200 Directional Crosshatch SS.

Classic ‘Directional' with carborundum tactile: Select from:

DIR0140 Directional Carborundum SS.

DIR0150 Directional Carborundum Black PVD SS.

Classic: Select from:

DIRP0750B Directional Black Urethane 5P.

DIRP0750LG Directional Light Grey Urethane 5P.

DIRP0750W Directional White Urethane 5P.

DIRP0750Y Directional Yellow Urethane 5P.

Integrated Tactile products

Classic ‘Warning' tactile: Select from:

DIT0020 Classic SS Integrated Plate 300 x 600 mm.

DIT0070 Classic SS Integrated Plate 100 x 300 mm.

DIT0080 Classic SS Integrated Plate 100 x 600 mm.

DIT0110 Black PVD on SS Integrated Plate 300 x 300 mm.

DIT0120 Black PVD on SS Integrated Plate 100 x 300 mm.

Classic ‘Warning' Black Top tactile: Select from:

DIT0030 Blacktop ‑ SS Integrated Plate 300 x 300 mm.

DIT0040 Blacktop ‑ SS Integrated Plate 300 x 600 mm.

DIT0090 Blacktop ‑ SS Integrated Plate 100 x 300 mm.

DIT0100 Blacktop ‑ SS Integrated Plate 100 x 600 mm.

Cross‑Hatch ‘Directional' grooved tactile: Select from:

DIT0300 Directional Crosshatch – SS Integrated Plate 300 x 300 mm.

Fibreglass Reinforced ‘Warning' tactile: Select from:

DFP001 Reinforced Fibreglass Warning 600 x 300 mm White.

DFP002 Reinforced Fibreglass Warning 600 x 300 mm Yellow.

DFP003 Reinforced Fibreglass Warning 600 x 300 mm Black.

Fibreglass Reinforced ‘Directional' tactile: Select from:

DFP011 Reinforced Fibreglass Directional 600 x 300 mm White.

DFP012 Reinforced Fibreglass Directional 600 x 300 mm Yellow.

DFP013 Reinforced Fibreglass Directional 600 x 300 mm Black.

Ultimat® Tactile: Select from:

DWU0010 Ultimat Warning Tactile Black 300 x 300 mm.

DWU0011 Ultimat Warning Tactile Yellow 300 x 300 mm.

DWU0012 Ultimat Warning Tactile Light Grey 300 x 300 mm.

DWU0014 Ultimat Warning Tactile White 300 x 300 mm.

Ultimat Duo® Tactile: Select from:

DWU0201 Ultimat Duo – Black TGSI on Grey Base 300 x 300 mm.

DWU0202 Ultimat Duo – Grey TGSI on Black Base 300 x 300 mm.

DWU0203 Ultimat Duo – Yellow TGSI on Black Base 300 x 300 mm.

DWU0204 Ultimat Duo – Black TGSI on Yellow Base 300 x 300 mm.

DWU0205 Ultimat Duo – White TGSI on Black Base 300 x 300 mm.

DWU0206 Ultimat Duo – Black TGSI on White Base 300 x 300 mm.

DWU0207 Ultimat Duo – White TGSI on Blue Base 300 x 300 mm.

DWU0208 Ultimat Duo – Blue TGSI on White Base 300 x 300 mm.

DWU0209 Ultimat Duo – White TGSI on Green Base 300 x 300 mm.

DWU0210 Ultimat Duo – Green TGSI on White Base 300 x 300 mm.

DWU0211 Ultimat Duo – Red TGSI on White Base 300 x 300 mm.

DWU0212 Ultimat Duo – White TGSI on Red Base 300 x 300 mm.

DWU0213 Ultimat Duo – Yellow TGSI on Blue Base 300 x 300 mm.

DWU0214 Ultimat Duo – Blue TGSI on Yellow Base 300 x 300 mm.

Carpet Tactile systems

Classic (Low/High Pile): Select from:

DT0303 Tactile Carpet Brass 3 mm.

DT0306 Tactile Carpet Brass 6 mm.

Terraced (Low/High Pile): Select from:

DT0113 Tactile Carpet Terraced SS 3 mm.

DT0116 Tactile Carpet Terraced SS 6 mm.

Ecotac®Classic (High Pile): Select from:

DT0125 Ecotac Carpet 25 mm Tactile Spike SS.

Steel Carpet Plates: Select from:

DCS0010 Steel Carpet Plate 100 x 300 mm.

DCS0020 Steel Carpet Plate 300 x 300 mm.

Note: A colour contrast may be required, in both wet and dry conditions, between the tactile indicators and the adjacent surface. The colour is to provide a luminance contrast to the surrounding surface to AS/NZS 1428.4.1 (2009) Appendix E. The NCC cites AS 1428.4 (1992) and AS/NZS 1428.4.1 (2009). The current edition is AS/NZS 1428.4.1 (2009).

Slip resistance classification: Refer to NATSPEC TECHnote DES 001, HB 197 (1999) and SA HB 198 (2014). Select the slip resistance test and classification to suit the location and application.

Substrate: Nominate the material of the substrate for fixing the tactile indicator.

#### Stair edging schedule

|  | SE1 | SE2 | SE3 |
| --- | --- | --- | --- |
| Product code |  |  |  |
| Application |  |  |  |
| Slip resistance classification |  |  |  |
| Substrate |  |  |  |

The codes in the header row of the schedule designate each application or location of the item scheduled. Edit the codes to match those in other contract documents.

Product code: Nominate the required product code from the options below. For multiple selections, edit table to suit.

Stair edging products

PEMKO® ‘Modular ‑ Carpet' edging: Select from:

DE0810B Modular – Carpet – Corduroy Black/Natural Anodised (3.2 m).

DE0810C Modular – Carpet – Corduroy Natural Anodised (3.2 m).

DE0820B Modular – Carpet – Pinstripe Black/Natural Anodised (3.2 m).

DE0820C Modular – Carpet – Pinstripe Natural Anodised (3.2 m).

DE0830B Modular – Carpet – Pleat Black/Natural Anodised (3.2 m).

DE0830C Modular – Carpet – Pleat Natural Anodised (3.2 m).

DE0840B Modular – Carpet – Stitch Black/Natural Anodised (3.2 m).

DE0840C Modular – Carpet – Stitch Natural Anodised (3.2 m).

PEMKO® ‘Modular ‑ Surface' edging: Select from:

DE0150B Modular – Surface – Corduroy Black/Natural Anodised (3.2 m).

DE0150C Modular –Surface – Corduroy Natural Anodised (3.2 m).

DE0250B Modular – Surface – Pinstripe Black/Natural Anodised (3.2 m).

DE0250C Modular – Surface – Pinstripe Natural Anodised (3.2 m).

DE0350B Modular – Surface – Pleat Black/Natural Anodised (3.2 m).

DE0350C Modular – Surface – Pleat Natural Anodised (3.2 m).

DE0450B Modular – Surface – Stitch Black/Natural Anodised (3.2 m).

DE0450C Modular – Surface – Stitch Natural Anodised (3.2 m).

PEMKO® Corduroy edging: Select from:

DE0100B Edge Corduroy 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0100C Edge Corduroy 50 x 3 mm (3.2 m). H/D Natural Anodised.

DE0100G Edge Corduroy 50 x 3 mm (3.2 m). Gold Anodised.

DE0101G Edge Corduroy 50 x 3 mm (2.5 m). ‘Bright' Gold Anodised.

DE0102G Edge Corduroy 50 x 3 mm (2.5 m). ‘Bronze' Gold Anodised.

DE0110B Edge Corduroy 10 x 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0110C Edge Corduroy 10 x 50 x 3 mm (3.2 m). H/D Natural Anodised.

DE0110G Edge Corduroy 10 x 50 x 3 mm (3.2 m). Gold Anodised.

PEMKO® Pinstripe edging: Select from:

DE0200B Edge Pinstripe 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0200C Edge Pinstripe 50 x 3 mm (3.2 m). H/D Natural Anodised.

DE0210B Edge Pinstripe 10 x 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0210C Edge Pinstripe 10 x 50 x 3 mm (3.2 m). H/D Natural Anodised.

PEMKO® Pleat edging: Select from:

DE0300B Edge Pleat 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0300C Edge Pleat 50 x 3 mm (2.5 m). H/D Natural Anodised.

DE0310B Edge Pleat 10 x 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0310C Edge Pleat 10 x 50 x 3 mm (3.2 m). H/D Natural Anodised.

PEMKO® Stitch edging: Select from:

DE0310B Edge Stitch 10 x 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0310C Edge Stitch 10 x 50 x 3 mm (3.2 m). H/D Natural Anodised.

DE0400B Edge Stitch 50 x 3 mm (3.2 m). H/D Black Anodised.

DE0400C Edge Stitch 50 x 3 mm (2.5 m). H/D Natural Anodised.

PEMKO® Suede edging: Select from:

DE0500B Edge Suede 55 x 3 mm (3.2 m). H/D Black Anodised.

DE0510B Edge Suede 10 x 55 x 3 mm (2.5 m). H/D Black Anodised.

DE0510C Edge Suede 10 x 55 x 3 mm (3.2 m). H/D Natural Anodised.

PEMKO® Corduroy Carpet edging: Select from:

DE0630B Carpet System Corduroy (3.2 m). H/D Black Anodised.

DE0630C Carpet System Corduroy (3.2 m). H/D Natural Anodised.

DE0630G Carpet System Corduroy (2.5 m). Gold Anodised.

DE0640C Carpet System Pinstripe (3.2 m). H/D Black Anodised.

DE0670C Carpet System Suede (3.2 m). H/D Natural Anodised.

PEMKO® Urban edging: Select from:

DE0680B Urban 10 x 58 x 5 mm (3.2 m). H/D Black Anodised.

DE0680C Urban 10 x 58 x 5 mm (3.2 m). H/D Natural Anodised.

DE0690B Urban 58 x 5 mm (3.2 m). H/D Black Anodised.

DE0690C Urban 58 x 5 mm (2.5 m). H/D Natural Anodised Grey.

DE0700B Urban 10 x 65 x 5 mm (3.2 m). H/D Black Anodised.

DE0700BMF Urban 10 x 65 x 5 mm (3.2 m). H/D Natural/Black Anodised.

DE0700C Urban 10 x 65 x 5 mm (3.2 m). H/D Natural anodised.

DE0700CY Urban 10 x 65 x 5 mm (3.2 m). H/D Natural/Yellow Anodised.

DE0700W Urban 10 x 65 x 5 mm (3.2 m). H/D Natural/White Anodised.

PEMKO® Rugged edging: Select from:

DEF0010B Pemko Rugged Fibreglass 10 x 70 mm (3.2 m). Black.

DEF0010W Pemko Rugged Fibreglass 10 x 70 mm (3.2 m). White.

DEF0010Y Pemko Rugged Fibreglass 10 x 70 mm (3.2 m). Yellow.

DEF0050B Pemko Rugged Fibreglass 50 x 3 mm (3.2 m). Black.

Application: e.g. Stair nosing to NCC requirements, or edging to finishes as detailed.

Note: A colour contrast may be required, in both wet and dry conditions, between the tactile indicators and the adjacent surface. The colour is to provide a luminance contrast to the surrounding surface to AS/NZS 1428.4.1 (2009) Appendix E. The NCC cites AS 1428.4 (1992) and AS/NZS 1428.4.1 (2009). The current edition is AS/NZS 1428.4.1 (2009).

Slip resistance classification: Refer to NATSPEC TECHnote DES 001, HB 197 (1999) and SA HB 198 (2014). Select the slip resistance test and classification to suit the location and application.

Substrate: Nominate the material of the substrate for fixing the stair edging.

#### Thresholds and transition strips schedule

|  | TS1 | TS2 | TS3 |
| --- | --- | --- | --- |
| Product code |  |  |  |

The codes in the header row of the schedule designate each application or location of the item scheduled. Edit the codes to match those in other contract documents.

Product code: Nominate the required product code from the options below. For multiple selections, edit table to suit.

Threshold and transition strip products

PEMKO® Threshold Plates : Select from:

DP0041 Pemko Threshold Plate 44 x 11 mm (1 m).

DP0042 Pemko Threshold Plate 44 x 11 mm (2 m).

DP0043 Pemko Threshold Plate 44 x 11 mm (3 m).

DP0131 Pemko Threshold Plate 25 x 5 mm (1 m).

DP0132 Pemko Threshold Plate 25 x 5 mm (2 m).

DP0191 Pemko Threshold Plate 100 x 13 mm (1 m).

DP0192 Pemko Threshold Plate 100 x 13 mm (2 m).

DP0193 Pemko Threshold Plate 100 x 13 mm (3 m).

DP0271 Pemko Threshold Plate 38 x 11 mm (1 m).

DP0272 Pemko Threshold Plate 38 x 11 mm (2 m).

DP0273 Pemko Threshold Plate 38 x 11 mm (3 m).

DP0281 Pemko Threshold Plate 127 x 13 mm (1 m).

DP0282 Pemko Threshold Plate 127 x 13 mm (2 m).

DP0283 Pemko Threshold Plate 127 x 13 mm (3 m).

DP0291 Pemko Threshold Plate 76 x 13 mm (1 m).

DP0292 Pemko Threshold Plate 76 x 13 mm (2 m).

DP0293 Pemko Threshold Plate 76 x 13 mm (3 m).

DP0661 Pemko Transition Strip 40 x 17 mm (1 m).

DP0662 Pemko Transition Strip 40 x 17 mm (2 m).

DP0663 Pemko Transition Strip 40 x 17 mm (3 m).

DP0771 Pemko Threshold Ramp 100 x 13 mm (1 m).

DP0771BK Pemko Threshold Ramp Black 100 x 13 mm (1 m).

DP0772 Pemko Threshold Ramp 100 x 13 mm (2 m).

DP0772BK Pemko Threshold Ramp Black 100 x 13 mm (2 m).

DP0773 Pemko Threshold Ramp 100 x 13 mm (3 m).

DP0773BK Pemko Threshold Ramp Black 100 x 13 mm (3 m).

DP0821 Pemko Threshold Plate 40 x 5 mm (1 m).

DP0822 Pemko Threshold Plate 40 x 5 mm (2 m).

DP0823 Pemko Threshold Plate 40 x 5 mm (3 m).

DP0951 Pemko Ribbed Threshold Plate 40 x 3 mm (1 m).

DP0951BK Pemko Ribbed Threshold Plate Black 40 x 3 mm (1 m).

DP0952 Pemko Ribbed Threshold Plate 40 x 3 mm (2 m).

DP0952BK Pemko Ribbed Threshold Plate Black 40 x 3 mm (2 m).

DP0953 Pemko Ribbed Threshold Plate 40 x 3 mm (3 m).

DP0953BK Pemko Ribbed Threshold Plate Black 40 x 3 mm (3 m).

DP0961 Pemko Threshold Plate 40 x 3 mm (1 m).

DP0962 Pemko Threshold Plate 40 x 3 mm (2 m).

DP0963 Pemko Threshold Plate 40 x 3 mm (3 m).

DP0981 Pemko Threshold Ramp 150 x 19 mm (1 m).

DP0982 Pemko Threshold Ramp 150 x 19 mm (2 m).

DP0983 Pemko Threshold Ramp 150 x 19 mm (3 m).

DP1121 Pemko Threshold Ramp 50 x 8 mm (1 m).

DP1122 Pemko Threshold Ramp 50 x 8 mm (2 m).

DP1123 Pemko Threshold Ramp 50 x 8 mm (3 m).

DP1151 Pemko Threshold Plate 76 x 8 mm (1 m).

DP1151BK Pemko Threshold Plate Black 76 x 8 mm (1 m).

DP1152 Pemko Threshold Plate 76 x 8 mm (2 m).

DP1152BK Pemko Threshold Plate Black 76 x 8 mm (2 m).

DP1153 Pemko Threshold Plate 76 x 8 mm (3 m).

DP1153BK Pemko Threshold Plate Black 76 x 8 mm (3 m).

DP1161 Pemko Threshold Plate 100 x 6 mm (1 m).

DP1161BK Pemko Threshold Plate Black 100 x 6 mm (1 m).

DP1162 Pemko Threshold Plate 100 x 6 mm (2 m).

DP1162BK Pemko Threshold Plate Black 100 x 6 mm (2 m).

DP1163 Pemko Threshold Plate 100 x 6 mm (3 m).

DP1163BK Pemko Threshold Plate Black 100 x 6 mm (3 m).

DP1371 Pemko Threshold Ramp 150  x 6 mm (1 m).

DP1372 Pemko Threshold Ramp 150 x 6 mm (2 m).

DP1373 Pemko Threshold Ramp 150 x 6 mm (3 m).

DP1381 Pemko Threshold Ramp 150 x 13 mm (1 m).

DP1382 Pemko Threshold Ramp 150 x 13 mm (2 m).

DP1383 Pemko Threshold Ramp 150 x 13 mm (3 m).

DP1511 Pemko Threshold Plate 125 x 6 mm (1 m).

DP1511BK Pemko Threshold Plate Black 125 x 6 mm (1 m).

DP1512 Pemko Threshold Plate 125 x 6 mm (2 m).

DP1512BK Pemko Threshold Plate Black 125 x 6 mm (2 m).

DP1513 Pemko Threshold Plate 125 x 6 mm (3 m).

DP1513BK Pemko Threshold Plate Black 125 x 6 mm (3 m).

#### Urban edge protectors and handrail tactile indicators schedule

|  | EH1 | EH2 | EH3 |
| --- | --- | --- | --- |
| Type |  |  |  |
| Product code |  |  |  |

The codes in the header row of the schedule designate each application or location of the item scheduled. Edit the codes to match those in other contract documents.

Type: Select from Urban Edge Protector or Handrail Tactile Indicator.

Product code: Nominate the required product code from the options below. For multiple selections, edit table to suit.

Urban Edge Protector products

Round button: Select from:

DEP0010 Edge Protector Button Radius SS.

DEP0300 Edge Protector Button Radius Brass.

Bevelled button: Select from:

DEP0020 Edge Protector Button Bevelled SS.

DEP00301 Edge Protector Button Bevelled Brass.

Thin bar: Select from:

DEP0040 Edge Protector Bar Thin SS.

Thick bar: Select from:

DEP0060 Edge Protector Bar Thick SS.

Bullnose: Select from:

DEP0080 Edge Protector Bullnose SS.

Handrail Tactile Indicator products

Handrail button: Select from:

DHB0010 Handrail Button SS.

REFERENCED DOCUMENTS

The following documents are incorporated into this worksection by reference:

AS 1428		Design for access and mobility

AS 1428.1	2021	General requirements for access - New building work

AS/NZS 1428.4.1	2009	Means to assist the orientation of people with vision impairment - Tactile ground surface indicators

AS 4586	2013	Slip resistance classification of new pedestrian surface materials

AS 4663	2013	Slip resistance measurement of existing pedestrian surfaces

The following documents are mentioned only in the Guidance text:

AS 1428		Design for access and mobility

AS 1428.1	2001	General requirements for access - New building work

AS 1428.4	1992	Tactile ground surface indicators for the orientation of people with vision impairment

HB 197	1999	An introductory guide to the slip resistance of pedestrian surface materials

SA HB 198	2014	Guide to the specification and testing of slip resistance of pedestrian surfaces

NCC A5G1	2025	Governing requirements - Documentation of design and construction - Suitability

NATSPEC DES 001		Slip resistance performance

NATSPEC GEN 006		Product specifying and substitution

NATSPEC GEN 024		Using NATSPEC selections schedules

NATSPEC TR 01		Specifying ESD